23 November 2011

TEST OF ANTENNA BALUNS

# Last week Stefan Wikander and Roland Sandberg tested antenna baluns. SW who has the proper equiment was the real executor and RS was only assisting providing SW with some baluns and giving good and probably also bad advice.

The Wellbrook UMG 130- kindly provided by OJS in Norway- was the reference balun to

which we compared other applications.

Only the signal transfer was tested- noise rejection which is the most important feature

was not tested- that has to be tested by A/B comparison. This in due time when RS gets

the relay box needed made- this will take some time. Could be next summer when there

are more time for that and much easier to make the comparison when it´s warm outside.

Here are the baluns that were tested along with their measured parameters

Toroid applications

Ferrite		Winding	Turns	Inductance		Kap.

Wellbrook UMG 130	Unk	Unk	263uH/2.75mH	30pf

Philips 3E25  25mm	OL	6/18	193uH/2.1Mh                       55pf

Amidon 82-75	OL	10/30	332uH/3.2mH	35pf

Amidon  114-77	OL	11/33	143uH/1.3mH	204pf

Amidon 73-202	OL	3/9	78uH/725uH		25pf

Amidon 114-75	OL	11/33	365uH/3.35mH	21pf

Philips 3H2 25mm	OL	8/24	120uH/1.16mH	15.8pf

Philips 3H2 25mm	SS 	6/18	74uH/755uH                         8pf

Philips 3F3 25mm	TW	6x4	61 uH/580uH	29pf

Test Results

12 MHz           1.7 MHz       0.5 MHz

Wellbrook 	UMG 130		  0  =ref.	0=ref.	           0=ref.

Philips 3E25 25mm  OL 6/18	+2db	+2db	           0db

Amidon 82-75  OL 10/30		-2db	+1db	           0db

Amidon 114-77  11/33		-6db	-2db	           0db

Amidon 73-202   3/9		+1db	 0db	         +1db

Amidon 114-75  OL 11/33		-2db	+2db	         +2db

Philips 3H2  25mm OL 8/24	+1db	+1db 	         +3db

Philips 3F3  25 mm TW 6x4	-3db	-2db	           0db

Philips 3H2 25mm SS 6/18		-9db	 0db	         +3db

Test antenna  “minibeverage”  ca 50 meters long grounded via 500 ohm to get the right impedance

Cable 50 meter RG58 CU  coax from rx. 5 grounding rods for the balun och 4 rods for termination.

OL= antenna winding beneath, coax winding above covering the whole toroid round.

TW= trifiliar winding + a separate antenna winding above

SS= antenna and coax winding on separate sides on the ferrite as far away from each other as possible

## Summary report

# Well-what to say…

Wellbrooks UMG 130 was a bit of a disappointment. If you aim for good MW reception

this is not the choice for you. And you have to pay about 50 euros to get them from England.

And you get 2 peaces of better ones from the right place or even 4 if you make them yourself.

Well- again the winner was Philips grey 3H2- not clearly better- but better. That is not just imagination- it gives a little bit more that the other ones if ou use it in the right way. It´s clearly a dedicated MW ferrite- if you go to 3..4 MHz it is useless. Perhaps we MW-addicted should advice Ferroxcube to take them back into production.

Amidon’s binocular BN 73-202 was the most linear along with 114-75 but it didn´t peak in MW.

Ferroxcube new 3E25 was also good and if you don´t have 3H2 available it’s a good choice.

But no application made a fool of itself and they are all capable to make good catches. The

differences are real marginal.

A much bigger role plays the capability to break the ground circuit and attenuate noise coming from our QTH and the power network via groundings. 3H2 has low capacity and that is probably the secret behind its good capability to attenuate noise.

But  noise attenuation has to be tested more thoroughly- let’s see next summer.

Stefan Wikander    Roland Sandberg